# EQUIPMENT AIRWORTHINESS & SAFETY STANDARDS

## General

### Purpose

The purpose of these standards is to ensure a certain minimum level of structural integrity and pilot safety in hang gliders of classes 1, 2, 4 and 5 and associated equipment.

### Change in glider configuration or construction

A glider shall fly throughout the championships as a single structural entity using the same standard of components used on the first day. Concessions to this rule are made to cover the case of essential repairs (see 2.16.4. Damage to a glider).

## Airworthiness Standards

All gliders and associated equipment shall be of sufficient performance and standard of airworthiness to meet the demands of international championships.

### Classification of Hang Gliders

Hang gliders permitted to fly in FAI sanctioned competitions must fall into one of the following categories. For 1st Category events the Local Regulations shall state which categories of glider are permitted:

- Certified Gliders – hang gliders of a make and model for which there is airworthiness approval issued by either the BHPA, DHV or HGMA and which have not been altered in any way since manufacture that would affect this certification. Sprogs must be set within the certified range.

- Uncertified Gliders – production model hang gliders which have not yet obtained airworthiness approval or certified models which have been altered from the certified configuration.

- Prototypes – development gliders produced by recognised manufacturers and flown by factory accredited pilots.

#### 12.1.2.1 A pilot flying a certified glider will be required to sign a statement that his or her glider is in certified compliance and hasn’t been altered in any way to take it out of certification. The certificate to be used is appended to this chapter.

12.1.2.2 Uncertified gliders will be allowed to fly in Cat. 1 competitions only if the pilot or manufacturer can produce pitch test results for the individual glider and/or a description of the changes made to a certified glider is presented with confirmation from the manufacturer that the changes do not detract from the airworthiness of the glider. Pitch test results must specify the sprog settings used during testing.

12.1.2.3 Prototype gliders are only allowed with the manufacturer’s statement that the pilot is approved to fly this glider. Furthermore, a statement from the manufacturer must be produced confirming that the glider is airworthy and specifying the sprog settings at which this confirmation is valid.

### Strength

Hang gliders must comply with the load test certification standards of, the HGMA, BHPA or DHV, or similar testing body.

Where dimensional limits are applied to structures, these have been chosen such that adequate strength is achievable with materials currently in use.

The standards in 12.3 below override the certified configuration of a glider.

## Structural limits

### Structural Cables

Minimum diameter of any structural external wire cables is 1.9 mm or 5/64 inches.

### Wire Attachment Points

Where an external compression strut is braced with rigging wires they must attach within 10cm of the point where the compression load is applied.

Side-wires shall attach to A-frames at no more than 10cm above the plane of the control tube, measured when the glider is resting on a horizontal surface.

Explanatory Notes: References to compression struts and rigging wires refer to the loads placed on parts of a glider by flight stresses. Gliders with cantilevered wings do not apply compression loads to the uprights, while in general, Class 1 gliders do have uprights which are under compression in flight.

Control cables are not deemed to be structural.

Any external part of the glider which has compression loads placed upon it during flight is an “external compression strut”, and therefore bracing wires attached to it shall conform to these rules.

Where the terminology or definitions which are used in these rules are in question with any particular glider, the relevant protest committee will provide a ruling.

### Control Bars (base tubes)

If a control bar is load bearing and made of materials other than metal, it must have an internal steel rigging cable that serves as a structural backup.  If a non-metallic base tube (control bar) does not show clear evidence of an internal rigging cable (end pins or vibration when tapped) the pilot must supply a manufacturer’s affidavit verifying the presence of a cable in the base tube.

### Pilot Suspension Systems

The pilot suspension must include a non-metallic load bearing material of minimum 50 mm2 cross-section area (normal material Nylon woven webbing with 1000kg breaking strain). The attachment loop must have a backup, which bypasses any mechanical devices and either the main, or backup must be non-metallic. If an integral (one piece) harness suspension/hook-in system is employed, the backup may have a mechanical link which allows it to loop around the keel and attach to itself independently of the primary system.

### Rescue Parachutes

A rescue parachute must be capable of deployment by both the right and left hand of the pilot in a normal flying attitude.

## Competing gliders

### Glider identification

Each competing glider must have a serial number for identification.

### Acceptance check.

All hang gliders must be made available to the organisers during the period of registration, for an acceptance check, in the configuration in which they will be flown. After the opening of the launch window on the first scheduled competition day no changes of hang glider may be made (see 2.16.4.).

### Airworthiness checks.

At any time during the championships the organisers and FAI officials have the right to inspect any competing glider and, if necessary, ground it for safety reasons.  They may also apply any other penalties listed in these rules and the Local Regulations for non-compliance with class or airworthiness standards.

## Pitch Stability

CIVL officials at Category 1 events can measure and record sprog settings on competing hang gliders or perform other inspections. All competing pilots should co-operate with the officials in order to collect safety data. This data will be used to provide an understanding of the current safety situation in hang gliding competitions.

## Ballast

### Limits

A competing glider may carry jettisonable ballast only in the form of fine sand or water. A pilot must avoid dropping ballast at any time or in a manner likely to affect other competing gliders or third parties.

The weight limit in Class 5 for all equipment (without glider), clothes and ballast is to be 25 Kg. Any pilot equipped with a second parachute can exceed this limitation by the value of the weight of his second parachute and its deployment system. The weight limit with an additional parachute is 3kg above the 25 kg limit. Weight can be measured at take-off or landing (bare foot with T shirt and trousers, then equipped) by the organisers at the request of the stewards or of the organisers.

In all cases, pilots must also comply with the weight limitations set by the manufacturer and the authority who provided the certificate of airworthiness;

Pilots not complying with those rules will be removed from the meet.

### Penalties

The normal penalty for non-compliance is a 20 % reduction in score for the last round flown, except where specified differently elsewhere in these regulations. If during a subsequent round the glider is again found to be non-compliant a 0 score will result for that round. At the discretion of the Meet Director a lesser penalty may be applied in rare cases due to extenuating circumstances.

Appendix to Section 7, Chapter 12

CERTIFIED GLIDER STATEMENT

I, the undersigned, declare that the Class ___ glider ___________________________ (make)                              __________________ (model) I will fly in the _______________________________ (Name of event) _____________________ Championship, from _________ to ___________ (dates) is a model certified by one or more of the internationally recognized certifying bodies (namely the DHV, HGMA or the BHPA).  Furthermore I declare that I have not altered the configuration of the glider since purchase and as far as I am aware it is in certified configuration and I undertake not to alter this configuration during the championship.  I understand that I am the sole individual responsible for the integrity of my glider and to the best of my knowledge it is damage free and airworthy.

Signed on this date: _____________________________

___________________________________________________________________________

Signature of Participant					          Printed name of Participant

Address of Participant: ________________________________________________________

___________________________________________________________________________

Signature of Witness					               	Printed name of Witness

Address of Witness: __________________________________________________________

If your glider is not a certified model or is not in certified configuration

DO NOT SIGN THIS STATEMENT

but instead comply with Section 7A 12.1.2.2 or 3